Automated Stitching Pipeline

User Manual & Developer Reference

v1.0  •  June 2026
By Twan Eurlings, Maastricht University, MSc Imaging Engineering

# Contents

1. Overview

2. Prerequisites & Installation

3. Critical Configuration — Lines to Edit Before Running

4. How to Launch the Application

5. Step-by-Step Walkthrough (GUI)

6. Code Architecture — Section-by-Section Explanation

7. Output Files

8. Possible Improvements & Known Limitations

# 1. Overview

AutomatedStitchingPipeline.py is a GUI application that guides users through the process of stitching together microscopy (or other) tile images into a single composite image using Fiji's built-in Grid/Collection Stitching plugin.

The application is built with Python's tkinter and communicates with Fiji via the pyimagej bridge. It presents a 6-step wizard where the user configures acquisition type, tile scan direction, grid dimensions, file paths, optional file renaming, and then launches the stitching job — all without writing any macro code.

## What the pipeline does

- Accepts a folder of numbered tile images (e.g. tile_1.tif … tile_20.tif).

- Runs Fiji's Grid/Collection Stitching macro via pyimagej.

- Merges three separate channel output files (c1, c2, c3) into an RGB composite.

- Saves the final result as stitched_RGB.tif in the output directory.

- Cleans up intermediate files automatically.

# 2. Prerequisites & Installation

## Required software

| Component | Details |
| --- | --- |
| Python | 3.8 or newer |
| Fiji / ImageJ2 | Download from https://fiji.sc — must be the full Fiji distribution, not plain ImageJ |
| Java (JDK) | Required by pyimagej; OpenJDK 11 or 17 recommended |
| pyimagej | pip install pyimagej scyjava |
| tkinter | Bundled with most Python distributions; on Linux: sudo apt install python3-tk |

## Installing Python dependencies

pip install pyimagej scyjava

If you are on Windows and pyimagej fails to initialise Fiji, make sure JAVA_HOME points to a valid JDK installation and that the JDK bin directory is on your PATH.

# 3. Critical Configuration — Lines to Edit Before Running

⚠️ ACTION REQUIRED The script contains a hard-coded Fiji path that must be changed to match your own machine. The application will crash on startup if this path does not exist.

## 3.1  Fiji installation path (line 14)

This is the single most important change before running the script.

# LINE 14 — CHANGE THIS to your own Fiji folder:

fiji_directory = "C:\\Users\\janeu\\Downloads\\fiji-latest-win64-jdk\\Fiji"

# Windows example:

fiji_directory = "C:\\Users\\YourName\\Downloads\\Fiji.app"

# macOS example:

fiji_directory = "/Applications/Fiji.app"

# Linux example:

fiji_directory = "/opt/Fiji.app"

## 3.2  Fallback path candidates (lines 507–516)

The _guess_fiji() function in StepPaths tries several default locations to pre-fill the GUI field. If your Fiji lives somewhere non-standard you may want to add your path to the candidates list:

def _guess_fiji(self):

candidates = [

"/Applications/Fiji.app/Contents/MacOS/ImageJ-macosx",

fiji_directory,   # <-- already uses line 14 value

"/opt/Fiji.app/ImageJ-linux64",

# Add your custom path here if needed:

# "D:\\Tools\\Fiji.app",

]

## 3.3  Output file channel names (lines 943–960)

The macro assumes Fiji outputs exactly three channel files named img_t1_z1_c1, img_t1_z1_c2, and img_t1_z1_c3. If your stitching job produces differently named or a different number of channel files, update lines 943–960:

open(output_dir + "/img_t1_z1_c1")   // channel 1

open(output_dir + "/img_t1_z1_c2")   // channel 2

open(output_dir + "/img_t1_z1_c3")   // channel 3

run("Merge Channels...", "c1=img_t1_z1_c1 c2=img_t1_z1_c2 c3=img_t1_z1_c3 create");

If you have a different number of channels or different naming conventions from your microscope software, change these open() calls and the Merge Channels arguments accordingly.

## 3.4  Stitching quality thresholds (lines 926–928)

These three values control how strictly Fiji validates tile alignments. The defaults are conservative and suitable for most well-acquired datasets. Adjust them if stitching produces artefacts or rejects valid overlaps:

"regression_threshold":           0.30,   # lower = more permissive

"max/avg_displacement_threshold": 2.50,   # higher = more permissive

"absolute_displacement_threshold": 3.50,  # higher = more permissive

## 3.5  Fusion method (line 925)

The fusion method is currently set to Linear Blending, which blends overlapping areas smoothly. To change it, modify:

"fusion_method": "[Linear Blending]",

# Other options supported by the Fiji plugin:

# "[Average]", "[Median]", "[Max. Intensity]", "[Min. Intensity]",

# "[Intensity of random input tile]", "[Do not fuse images (only write TileConfiguration)]"

## 3.6  Computation mode (line 930)

"computation_parameters": "[Save memory (but be slower)]",

# Uncomment for faster stitching if RAM allows:

# "computation_parameters": "[Safe computation time (but use more RAM)]",

# 4. How to Launch the Application

# Navigate to the folder containing the script:

cd /path/to/script

# Run:

python AutomatedStitchingPipeline.py

ℹ️ Startup time On first launch Fiji initialises via pyimagej, which can take 30–90 seconds depending on your hardware. The console will print progress messages. The GUI window will appear once initialisation completes.

# 5. Step-by-Step Walkthrough (GUI)

The wizard has six steps accessible via the Next/Back buttons at the bottom of the window. A progress bar at the top shows the current step.

## Step 1 — Acquisition Type

Choose how the microscope camera scanned across the sample. Each option shows an animated preview of the scan pattern:

| Option | When to use |
| --- | --- |
| Grid: row-by-row | Camera moved left-to-right (or right-to-left) across each row, then stepped down |
| Grid: column-by-column | Camera moved top-to-bottom down each column, then stepped sideways |
| Grid: snake by rows | Camera zigzagged: row 1 left→right, row 2 right→left, etc. |
| Grid: snake by columns | Camera zigzagged: column 1 top→bottom, column 2 bottom→top, etc. |
| Unknown positions | You do not know the scan order — Fiji will try to detect it automatically (slower) |

## Step 2 — Start Corner & Direction

Select which corner of the sample the scan started from. This step is skipped (shown as not applicable) if you chose Unknown positions in Step 1.

## Step 3 — Grid Dimensions & Overlap

| Field | Description |
| --- | --- |
| Columns | Number of tile columns in the grid (whole number ≥ 1) |
| Rows | Number of tile rows in the grid (whole number ≥ 1) |
| Overlap (%) | Estimated percentage overlap between adjacent tiles (0–80). Check your microscope acquisition settings if unsure; 10–20% is typical. |

## Step 4 — File Paths

| Field | Description |
| --- | --- |
| Fiji executable | Path to the Fiji/ImageJ executable. Pre-filled from the value on line 14. Use Browse to change. |
| Tile image folder | Folder containing all numbered tile images. Use Browse to select. |
| Output folder | Where the stitched result will be written. Will be created if it does not exist. |
| File name pattern | Naming template using {i} as the tile index, e.g. tile_{i}.tif or img_{i}.tiff |
| First tile index | The number in the filename of the very first tile (often 0 or 1) |

## Step 5 — Rename Tile Images (Optional)

Use this step if your tile filenames do not already follow a consistent numbered pattern required by Fiji. The tool will:

- Sort all image files in the selected folder alphabetically.

- Rename them to basename_N.ext (e.g. tile_1.tif, tile_2.tif, …).

- Warn you before overwriting any existing files.

This step is independent of the stitching run and does not affect settings from the other steps. Supported image formats: .tif, .tiff, .png, .jpg, .jpeg, .bmp.

## Step 6 — Run Stitching

A summary of all your chosen settings is displayed. Click ▶ Run Fiji stitching to start. Progress and errors appear in the dark log console below the button. When complete the button changes to ▶ Run again.

The “← Change settings” link takes you back to Step 1 at any time.

🐍 Easter egg A small snake game is hidden on Step 6. Click the snake emoji (🐍) in the top-right corner to play.

# 6. Code Architecture — Section-by-Section Explanation

## Imports & Fiji initialisation (lines 1–18)

Imports the standard library modules (tkinter, subprocess, threading, os, sys, random) and pyimagej. Fiji is initialised immediately on startup at module level:

fiji_directory = "C:\\Users\\janeu\\Downloads\\fiji-latest-win64-jdk\\Fiji"

ij = imagej.init(fiji_directory)

Because this runs before the GUI window opens, any error here (wrong path, missing Java, etc.) will crash the script before any window appears. All console print statements throughout the file are startup diagnostics.

## Colour palette & card helper (lines 20–52)

Defines all colour constants used by the UI (BG, PANEL, BORDER, ACCENT, etc.) and a small card() factory function that creates a flat white panel with a light border — the visual container used in every wizard step.

## Path generators (lines 59–118)

Five pure functions that produce an ordered list of (row, col) coordinate pairs representing each tile's position in the grid. These are used by the animated TilePreview canvas:

| Function | Pattern produced |
| --- | --- |
| make_row_path() | Row by row, respecting start corner from direction string |
| make_col_path() | Column by column |
| make_snake_row_path() | Zigzag across rows (boustrophedon) |
| make_snake_col_path() | Zigzag across columns |
| make_unknown_path() | Random shuffle of all cells (visualises unknown mode) |

## TilePreview canvas (lines 125–196)

A tkinter Canvas subclass that animates a small grid of coloured tiles to show the scan order. A “head” tile advances one step every 100 ms, leaving a short trail behind it. Three visual styles exist: linear (blue), snake (green), and unknown (purple). Every wizard card that shows a scan-order option embeds one of these previews.

## DIRECTIONS data & _path_for() helper (lines 203–242)

DIRECTIONS is a dictionary keyed by acquisition type, mapping to a list of (direction_name, human_description) tuples used to populate the Step 2 radio buttons. _path_for() maps a type + direction combination to the correct path-generator function and preview style.

## StepType — Step 1 (lines 250–314)

A tk.Frame that renders the acquisition-type selection cards. Each card contains a radio button, label text, and a live TilePreview. Selecting a card highlights it with a blue border.

## StepDirection — Step 2 (lines 321–397)

Similar to StepType but dynamically built: the available direction options depend on the acquisition type chosen in Step 1. The load_for_type() method is called by the wizard when advancing from Step 1, rebuilding all radio button cards to match. If Unknown positions was chosen, the step shows only a static label.

## StepGrid — Step 3 (lines 403–451)

Three text entry fields (Columns, Rows, Overlap). validate() enforces that columns and rows are whole numbers ≥ 1 and that overlap is between 0 and 80.

## StepPaths — Step 4 (lines 457–544)

Five fields for Fiji path, input folder, output folder, file name pattern, and first index. The Fiji field is pre-filled by _guess_fiji(), which checks a list of standard locations. The input/output fields have Browse buttons. validate() checks that all required fields are filled and warns if the file pattern lacks the {i} placeholder.

## SnakeGame easter egg (lines 551–636)

A fully playable Snake game implemented as a tk.Toplevel window. Use the arrow keys to move; press R to restart after a game over. It serves no functional purpose in the pipeline.

## StepRename — Step 5 (lines 641–810)

A self-contained file-renaming utility. Collects image files from a folder, sorts them alphabetically, and renames them sequentially to basename_N.ext. It performs a conflict-check pass before touching any files and logs all rename operations to the dark console widget at the bottom of the step. This step does not pass any data forward to Step 6.

## StepRun — Step 6 (lines 816–970)

The execution step. load_summary() is called when the wizard advances to this step and populates a summary card with all chosen settings. _run() launches a background thread (threading.Thread) that:

- Builds the args dictionary for the Fiji macro.

- Calls ij.py.run_macro() with a multi-line ImageJ macro string that runs Grid/Collection Stitching, opens the three channel output files, merges them into an RGB stack, saves stitched_RGB.tif, and closes the image.

- Removes the four intermediate files (TileConfiguration.txt, TileConfiguration.registered.txt, img_t1_z1_c1/c2/c3).

- Handles ImportError (pyimagej not installed) and general exceptions with coloured log output.

The thread is non-daemon (daemon=False), meaning the process will not exit until stitching finishes even if the window is closed.

## FijiWizard — main window (lines 975–1122)

The root tk.Tk window. Builds the progress-bar header, the scrollable page container, all six step frames, and the Back/Next navigation bar. _collect_cfg() gathers the get_values() dictionary from all steps (except StepRun) into a single config object that is passed to StepRun.load_summary().

# 7. Output Files

| File | Description |
| --- | --- |
| stitched_RGB.tif | Final merged RGB composite of the stitched tile grid. This is the main deliverable. |
| img_t1_z1_c1/c2/c3 | Per-channel stitched images produced by Fiji. Automatically deleted after merging. |
| TileConfiguration.txt | Tile layout written by Fiji before registration. Automatically deleted. |
| TileConfiguration.registered.txt | Refined tile layout after registration. Automatically deleted. |

# 8. Possible Improvements & Known Limitations

## High priority

- Remove the hard-coded fiji_directory from the top level and move Fiji initialisation inside StepRun._run() (inside the worker thread). This would eliminate the long startup freeze and allow the GUI to open immediately.

- Add a channel count selector in Step 4 (or Step 6) so users can specify how many channel files Fiji produces (1, 2, 3, 4, …) instead of hardcoding 3.

- Show a progress indicator during stitching — the log console currently goes silent during the pyimagej call, giving no feedback on a long run.

## Usability

- Remember last-used paths between sessions using Python's shelve or json config file.

- Add drag-and-drop support for the tile image folder.

- Allow the user to preview a subset of tiles before committing to the full stitch.

- Add a Help tooltip or question-mark icon on each step to explain microscope-specific terms.

## Robustness

- Check that the number of image files in the input folder matches grid_x × grid_y before launching, and warn the user if they do not match.

- Allow intermediate files (TileConfiguration, per-channel images) to be optionally retained for debugging.

- Add error recovery: if merging fails, do not delete the channel files so the user can recover manually.

- The os.remove() calls on lines 957–961 will raise FileNotFoundError if Fiji did not produce the expected files; these should be wrapped in try/except.

## Platform & compatibility

- Test on macOS and Linux; the path-guessing logic covers these platforms but the main fiji_directory constant is Windows-only.

- Support single-channel (greyscale) acquisitions by detecting whether Fiji outputs c1 only and skipping the merge step.

- Consider packaging as a standalone executable with PyInstaller so users without a Python environment can run it.

## Known limitations

- Only 3-channel RGB output is supported. Single-channel or more-than-3-channel datasets require manual code changes (see Section 3.3).

- The snake game easter egg has no pause button and will keep running even after the window loses focus.

- The Rename step and the Paths step use separate folder pickers; the user must remember to set the same folder in both places if they use renaming.

## Questions

In case of questions about the code, I am reachable on twan.eurlings@gmail.com
Please cite the use of this code via:
Eurlings, AJM (2026). Tile stitching pipeline [Source code]. GitHub. https://github.com/example/project